CoffCo

Reliability-Centred Maintenance Study

RCM2 Failure Modes and Effects Analysis

COFFCO-RCM-001  |  Issue 1.0

Commercial in Confidence

| Reference: | COFFCO-RCM-001 |
| --- | --- |
| Issue: | 1.0 |
| Date: | June 2026 |
| Methodology: | RCM2 (Moubray, 1997) — SAE JA1011 / JA1012 compliant |
| System: | CoffCo In-Room Hotel Coffee Machine |
| Prepared by: | School of Systems Engineering (secondment) |
| Approved by: | Steve — Systems Design Authority |
| Parent documents: | COFFCO-SRS-001, COFFCO-ARCH-001 v2.0, COFFCO-SDD-001, COFFCO-FMECA-001 |

# 1.0  Introduction

## 1.1  Purpose

This document presents a Reliability-Centred Maintenance (RCM2) study for the CoffCo in-room hotel coffee machine. The study applies the RCM2 methodology developed by John Moubray (1997), consistent with SAE JA1011 and JA1012, to determine the appropriate maintenance strategy for each failure mode across all six physical subsystems.

The primary outputs of this study are: a structured FMEA worksheet for each subsystem; a consequence category for each failure mode; and a recommended maintenance task — or a justified decision to run to failure — for each failure mode. Together these outputs constitute a defensible, evidence-based maintenance programme for the CoffCo system.

## 1.2  RCM2 Methodology Summary

RCM2 addresses seven questions for every function in the system:

- What are the functions and associated performance standards of the asset in its present operating context?

- In what ways can it fail to fulfil its functions? (Functional failures)

- What causes each functional failure? (Failure modes)

- What happens when each failure occurs? (Failure effects)

- In what way does each failure matter? (Failure consequences)

- What should be done to predict or prevent each failure? (Proactive tasks)

- What should be done if a suitable proactive task cannot be found? (Default actions)

## 1.3  Consequence Categories

RCM2 classifies each failure mode into one of four consequence categories, which determine the logical order of task selection:

| Cat | Category | Definition |
| --- | --- | --- |
| A | Safety / Environmental — Hidden | Failure mode is hidden from operating crew AND could cause injury, death, or environmental harm. Mandatory failure-finding task required. |
| B | Safety / Environmental — Evident | Failure mode is evident to operating crew AND could cause injury, death, or environmental harm. On-condition or scheduled task required. |
| C | Operational — Evident | Failure has direct adverse effect on operational capability (availability, output, guest service). Economic justification for preventive task. |
| D | Non-Operational — Evident | Failure has no direct effect on safety or operations. Preventive task justified only if cheaper than reactive repair. |

## 1.4  Task Types

| Code | Task Type | Description |
| --- | --- | --- |
| OC | On-Condition | Monitor the item for evidence of potential failure; task is justified if the interval between potential failure and functional failure (P-F interval) is long enough to act. Preferred task type. |
| SR | Scheduled Restoration | Restore an item to a defined standard at a fixed interval, regardless of its condition. Justified where condition cannot be monitored effectively. |
| SD | Scheduled Discard | Replace an item at a fixed interval regardless of condition. Justified for items with a defined safe life or where failure consequences are severe. |
| FF | Failure Finding | Periodically test a hidden function to detect whether it has already failed. Mandatory for all hidden failures with safety consequences (Category A). |
| RTF | Run to Failure | No preventive task is justified; accept failure and restore. Appropriate only for Category C/D failures where prevention costs more than restoration. |

## 1.5  Operating Context

| Environment: | Heated hotel room, 18–25°C ambient, 30–70% RH. Mains supply 110–240V AC (EU/UK/US). No user technical knowledge assumed. |
| --- | --- |
| Duty cycle: | 2 brews per day average; up to 4 peak. 365 days per year. Continuous standby when not brewing. |
| Maintenance access: | Daily housekeeping (non-technical): consumable restock, visual check, drip tray. Annual field engineer: full maintenance visit. Remote: cloud ops telemetry continuous. |
| MTBF target: | ≥26,280 hours (3 years continuous operation at duty cycle). Maintenance interval target ≥365 days. |
| Safety standards: | IEC 60335 (electrical safety), IEC 61140 Class II (double insulation), EN 55014 Class B (EMC), HACCP (food safety). |
| Consumable: | Implementation-neutral. Water reservoir ≥2.5L. Milk refrigerated ≤4°C. Coffee consumable: format per TD-01 (TBD). |

# 2.0  Failure Mode Summary

The table below provides a consolidated view of all 22 failure modes across six subsystems, their consequence categories, and primary task types. Full analysis for each subsystem follows in Section 3.

| ID | SS | Failure Mode | Sev | Occ | Cat | Task |
| --- | --- | --- | --- | --- | --- | --- |
| FM-01 | SS-2/SS-3 | Heating element failure | Sev: 7/10 | Occ: 3/10 | C | OC |
| FM-02 | SS-2/SS-3 | Thermal cutoff failure | Sev: 9/10 | Occ: 2/10 | B | FF |
| FM-03 | SS-2/SS-3 | Pump wear | Sev: 5/10 | Occ: 5/10 | D | SR |
| FM-04 | SS-2/SS-3 | Pressure relief valve failure | Sev: 8/10 | Occ: 2/10 | B | OC |
| FM-05 | SS-2/SS-3 | Frother heater degradation | Sev: 7/10 | Occ: 3/10 | C | OC |
| FM-06 | SS-2/SS-3 | Refrigeration unit failure | Sev: 9/10 | Occ: 2/10 | B | FF |
| FM-07 | SS-2/SS-3 | Fluid level sensor failure | Sev: 10/10 | Occ: 1/10 | A | FF |
| FM-08 | SS-4 | PSU component failure (capacitor, transformer, rectifie | Sev: 6/10 | Occ: 2/10 | C | RTF |
| FM-09 | SS-4 | Insulation breakdown | Sev: 10/10 | Occ: 1/10 | A | FF |
| FM-10 | SS-4 | Clip mechanism wear | Sev: 6/10 | Occ: 3/10 | C | OC |
| FM-11 | SS-4 | Firmware error retaining subsystems active in standby | Sev: 5/10 | Occ: 2/10 | D | OC |
| FM-12 | SS-2/SS-5 | Firmware corruption | Sev: 6/10 | Occ: 2/10 | C | OC |
| FM-13 | SS-2/SS-5 | Sensor wire break | Sev: 9/10 | Occ: 2/10 | A | FF |
| FM-14 | SS-2/SS-5 | Wireless module failure | Sev: 7/10 | Occ: 3/10 | A | FF |
| FM-15 | SS-2/SS-5 | Firmware logic error | Sev: 10/10 | Occ: 1/10 | A | FF |
| FM-16 | SS-1 | Button | Sev: 5/10 | Occ: 3/10 | C | OC |
| FM-17 | SS-1 | LED | Sev: 6/10 | Occ: 2/10 | A | FF |
| FM-18 | SS-3 | Reservoir capacity insufficient | Sev: 4/10 | Occ: 4/10 | C | OC |
| FM-19 | SS-3 | Filter media saturation | Sev: 6/10 | Occ: 6/10 | D | SR |
| FM-20 | SS-3 | Housekeeping failure to empty tray | Sev: 5/10 | Occ: 4/10 | C | FF |
| FM-21 | SS-6 | Anti-theft anchor fatigue | Sev: 4/10 | Occ: 3/10 | D | OC |
| FM-22 | SS-6 | Physical impact damage to casing | Sev: 9/10 | Occ: 2/10 | B | OC |
| FM-23 | SS-3 | Pump debris clogging (strainer absent) | Sev: 6/10 | Occ: 5/10 | C | SR |
| FM-24 | SS-2/SS-5 | NTC holder perforation (PP material) | Sev: 8/10 | Occ: 3/10 | A | FF |
| FM-25 | SS-2/SS-3 | Fluid path seal material degradation | Sev: 7/10 | Occ: 4/10 | B | SR |

Safety-Critical Failure Modes (Category A — Hidden, Safety Consequence) FM-07 (Dry-fire interlock), FM-09 (PSU insulation), FM-13 (Sensor monitoring), FM-14 (Telemetry — hidden), FM-15 (Interlock logic), FM-17 (Status display hidden fault): all require mandatory annual failure-finding tasks. These tasks cannot be deferred or omitted without formal SDA waiver and documented risk acceptance.

# 3.0  Subsystem RCM2 Analysis

## SS-2/SS-3 — Thermal Assembly

Functions in scope:

- F1: Heat water to 60–95°C for americano extraction

- F2: Maintain extraction pressure at 0.9–1.3 bar

- F3: Heat milk to 60–90°C and froth to stable foam

- F4: Refrigerate milk store to ≤4°C

- F5: Prevent heating element activation with empty fluid store (dry-fire interlock)

| FM-01 ¦ F1: Heat water to 60–95°C ¦ Cat C ¦ OC |  |
| --- | --- |
| Functional Failure | FF-01: Fails to heat water to minimum 60°C |
| Failure Mode | Heating element failure / thermostat drift — element does not reach target temperature |
| Local Effect | No hot water produced; extraction fails |
| System Effect | Machine enters Fault state; no americano dispensed; guest service failure |
| Hidden / End Effect | None — fault state indicated on UI |
| Severity / Occurrence | Severity: 7/10 ¦ Occurrence: 3/10 ¦ Detectability (unaided): 2/10 |
| Consequence Category | Category C — Operational — Evident |
| Task Type | OC — On-Condition |
| Recommended Task | On-condition: temperature sensor monitoring via cloud telemetry; alert if outlet temperature <58°C over 3 consecutive brews. Inspect heating element and thermostat annually. |
| Task Interval | Continuous telemetry + annual inspection |

| FM-02 ¦ F1: Heat water to 60–95°C ¦ Cat B ¦ FF |  |
| --- | --- |
| Functional Failure | FF-02: Heats water above maximum safe limit (>95°C) |
| Failure Mode | Thermal cutoff failure / control firmware error — element does not de-energise at target |
| Local Effect | Fluid exceeds 95°C; pressure risk; burn hazard at outlet |
| System Effect | Independent thermal cutoff activates; machine enters Fault state. If cutoff also fails: steam release, potential casing damage, burn risk to user |
| Hidden / End Effect | Secondary thermal cutoff failure is hidden until primary failure exposes it |
| Severity / Occurrence | Severity: 9/10 ¦ Occurrence: 2/10 ¦ Detectability (unaided): 3/10 |
| Consequence Category | Category B — Safety / Environmental — Evident |
| Task Type | FF — Failure Finding |
| Recommended Task | Failure-finding: test secondary thermal cutoff function at every scheduled maintenance visit (≥annually). On-condition: telemetry monitors outlet temperature; alert if >92°C. Scheduled discard: replace thermal cutoff at 3-year interval regardless of condition. |
| Task Interval | FF: annual ¦ OC: continuous ¦ SD: 3 years |

| FM-03 ¦ F2: Maintain 0.9–1.3 bar extraction pressure ¦ Cat D ¦ SR |  |
| --- | --- |
| Functional Failure | FF-03: Pressure falls below 0.9 bar during extraction |
| Failure Mode | Pump wear / blocked filter / scale accumulation reducing flow |
| Local Effect | Under-extraction; weak coffee; guest quality complaint |
| System Effect | No fault state — machine completes brew at reduced quality |
| Hidden / End Effect | None evident to user beyond taste quality |
| Severity / Occurrence | Severity: 5/10 ¦ Occurrence: 5/10 ¦ Detectability (unaided): 7/10 |
| Consequence Category | Category D — Non-Operational — Evident |
| Task Type | SR — Scheduled Restoration |
| Recommended Task | Scheduled restoration: descale programme every 90 days (scale is primary driver per FMECA). On-condition: pump current monitoring — rising current at constant flow indicates scale. Replace pump at 2-year interval or at first confirmed pressure fault. |
| Task Interval | SR: 90-day descale ¦ OC: continuous ¦ replace: 2 years |

| FM-04 ¦ F2: Maintain 0.9–1.3 bar extraction pressure ¦ Cat B ¦ OC |  |
| --- | --- |
| Functional Failure | FF-04: Pressure exceeds 1.3 bar (over-pressure) |
| Failure Mode | Pressure relief valve failure / blocked outlet / pump control fault |
| Local Effect | Over-extraction; bitter coffee; excess pressure in fluid path |
| System Effect | Risk of seal failure and hot fluid release; burn hazard; potential Fault state entry |
| Hidden / End Effect | None — seal failure or high-pressure outlet would be immediately apparent |
| Severity / Occurrence | Severity: 8/10 ¦ Occurrence: 2/10 ¦ Detectability (unaided): 3/10 |
| Consequence Category | Category B — Safety / Environmental — Evident |
| Task Type | OC — On-Condition |
| Recommended Task | On-condition: pressure sensor monitoring at outlet; alert if >1.25 bar (pre-fault threshold). Inspect pressure relief valve annually. Replace relief valve at 3-year interval (scheduled discard). |
| Task Interval | OC: continuous ¦ annual inspection ¦ SD: 3 years |

| FM-05 ¦ F3: Heat milk to 60–90°C and froth ¦ Cat C ¦ OC |  |
| --- | --- |
| Functional Failure | FF-05: Milk temperature outside 60–90°C band at dispensation |
| Failure Mode | Frother heater degradation / milk sensor drift / refrigeration to frother thermal path error |
| Local Effect | Milk too cold (poor froth) or too hot (scalded; >90°C burn risk at outlet) |
| System Effect | Guest quality complaint (cold); burn risk and HACCP breach (hot). Fault state if sensor detects >90°C. |
| Hidden / End Effect | Cold side: not always detected — guest receives poor froth with no fault indication |
| Severity / Occurrence | Severity: 7/10 ¦ Occurrence: 3/10 ¦ Detectability (unaided): 4/10 |
| Consequence Category | Category C — Operational — Evident |
| Task Type | OC — On-Condition |
| Recommended Task | On-condition: milk outlet temperature sensor monitored via telemetry. Alert if <58°C or >88°C. Inspect and calibrate milk temperature sensor annually. Clean frother nozzle after every 30 brew cycles (scheduled restoration). |
| Task Interval | OC: continuous ¦ frother clean: 30 cycles ¦ annual calibration |

| FM-06 ¦ F4: Refrigerate milk to ≤4°C ¦ Cat B ¦ FF |  |
| --- | --- |
| Functional Failure | FF-06: Milk store temperature rises above 4°C |
| Failure Mode | Refrigeration unit failure / ambient temperature extreme / door seal degradation |
| Local Effect | Milk above safe storage temperature; HACCP breach; food safety risk |
| System Effect | Machine should detect via milk store sensor and prevent milk dispensation. If sensor fails too: guest served spoiled milk; potential food safety incident. |
| Hidden / End Effect | Sensor failure concurrent with refrigeration failure is a hidden double failure |
| Severity / Occurrence | Severity: 9/10 ¦ Occurrence: 2/10 ¦ Detectability (unaided): 3/10 |
| Consequence Category | Category B — Safety / Environmental — Evident |
| Task Type | FF — Failure Finding |
| Recommended Task | On-condition: milk store temperature monitored continuously; alert to cloud ops if >4°C for >15 minutes. Failure-finding: test milk temperature sensor independently at each maintenance visit. Inspect door seals annually. |
| Task Interval | OC: continuous ¦ FF: annual ¦ seal inspection: annual |

| FM-07 ¦ F5: Dry-fire prevention interlock ¦ Cat A ¦ FF |  |
| --- | --- |
| Functional Failure | FF-07: Interlock fails to prevent heating element activation with empty fluid store |
| Failure Mode | Fluid level sensor failure / firmware logic error — brew commanded and heating activates with no water |
| Local Effect | Heating element activated with no cooling water — immediate overheating |
| System Effect | Heating element burnout; risk of fire; casing damage; potential thermal injury. Complete machine loss. |
| Hidden / End Effect | Interlock function is hidden — only revealed when fluid store is empty AND brew is commanded |
| Severity / Occurrence | Severity: 10/10 ¦ Occurrence: 1/10 ¦ Detectability (unaided): 2/10 |
| Consequence Category | Category A — Safety / Environmental — Hidden |
| Task Type | FF — Failure Finding |
| Recommended Task | Failure-finding: test dry-fire interlock function at every maintenance visit (annual minimum) — drain fluid store, command brew, confirm heating element does not activate and fault state is entered within 5 seconds. This is COFFCO-TC-008. On-condition: fluid level sensor monitored continuously via telemetry. |
| Task Interval | FF: annual (mandatory — safety-critical) ¦ OC: continuous |

## SS-4 — Power Supply Unit

Functions in scope:

- F1: Accept 110–240V AC input and supply regulated DC power to all subsystems

- F2: Provide electrical isolation between mains supply and user-accessible parts (IEC 61140 Class II)

- F3: Accept field-interchangeable regional plug module (EU/UK/US)

- F4: Limit standby power consumption to ≤0.5W

| FM-08 ¦ F1: Supply regulated DC power ¦ Cat C ¦ RTF |  |
| --- | --- |
| Functional Failure | FF-08: PSU fails to supply power (no output) |
| Failure Mode | PSU component failure (capacitor, transformer, rectifier) / mains supply interruption |
| Local Effect | All subsystems lose power; machine non-functional |
| System Effect | Machine unavailable; guest service failure; cloud ops alert on loss of telemetry heartbeat |
| Hidden / End Effect | Loss of telemetry heartbeat reveals outage to cloud ops within 60 seconds |
| Severity / Occurrence | Severity: 6/10 ¦ Occurrence: 2/10 ¦ Detectability (unaided): 2/10 |
| Consequence Category | Category C — Operational — Evident |
| Task Type | RTF — Run to Failure |
| Recommended Task | Run-to-failure: PSU is a COTS module; failure rate low and failure is immediately evident. Replace PSU module on failure. Cloud ops telemetry loss alert enables rapid field engineer dispatch. No scheduled maintenance task justified. |
| Task Interval | RTF — replace on failure |

| FM-09 ¦ F2: Electrical isolation (Class II) ¦ Cat A ¦ FF |  |
| --- | --- |
| Functional Failure | FF-09: Insulation failure — mains voltage accessible at user-touchable surface |
| Failure Mode | Insulation breakdown / mechanical damage to casing / ingress of conductive liquid |
| Local Effect | Mains voltage on accessible surface; immediate electrocution risk to user or housekeeping |
| System Effect | Potentially fatal injury. Regulatory non-compliance. Hotel liability. |
| Hidden / End Effect | Insulation degradation is hidden — only revealed on contact |
| Severity / Occurrence | Severity: 10/10 ¦ Occurrence: 1/10 ¦ Detectability (unaided): 1/10 |
| Consequence Category | Category A — Safety / Environmental — Hidden |
| Task Type | FF — Failure Finding |
| Recommended Task | Failure-finding: insulation resistance test (hipot) at every maintenance visit (annual). Inspect casing for mechanical damage and ingress at every housekeeping visit (daily). IEC 60335 certification provides design-level protection. Replace machine immediately on any detected insulation fault. |
| Task Interval | FF: annual hipot ¦ visual inspection: daily (housekeeping) |

| FM-10 ¦ F3: Regional plug module ¦ Cat C ¦ OC |  |
| --- | --- |
| Functional Failure | FF-10: Plug module fails to engage securely or detaches in service |
| Failure Mode | Clip mechanism wear / incorrect fitment / physical damage to plug module |
| Local Effect | Intermittent or lost mains connection; machine non-functional or intermittent |
| System Effect | Machine unavailable; arcing risk if partial contact maintained under load |
| Hidden / End Effect | None — loss of power is immediately evident |
| Severity / Occurrence | Severity: 6/10 ¦ Occurrence: 3/10 ¦ Detectability (unaided): 2/10 |
| Consequence Category | Category C — Operational — Evident |
| Task Type | OC — On-Condition |
| Recommended Task | On-condition: inspect plug module engagement and clip condition at each maintenance visit. Replace plug module if clip shows wear or play is felt. Scheduled discard at 5-year interval regardless of condition. |
| Task Interval | OC: annual inspection ¦ SD: 5 years |

| FM-11 ¦ F4: Standby power ≤0.5W ¦ Cat D ¦ OC |  |
| --- | --- |
| Functional Failure | FF-11: Standby power consumption exceeds 0.5W |
| Failure Mode | Firmware error retaining subsystems active in standby / component drift in PSU efficiency |
| Local Effect | Excess standby power; energy efficiency rating non-compliant; EU Ecodesign regulation breach |
| System Effect | Regulatory non-compliance across hotel estate; potential product recall or removal from sale |
| Hidden / End Effect | Not immediately evident — only detected by measurement |
| Severity / Occurrence | Severity: 5/10 ¦ Occurrence: 2/10 ¦ Detectability (unaided): 8/10 |
| Consequence Category | Category D — Non-Operational — Evident |
| Task Type | OC — On-Condition |
| Recommended Task | On-condition: standby power monitored via cloud telemetry (current sensing at PSU). Alert if measured standby power >0.4W (pre-threshold). Verify at annual maintenance visit using calibrated power meter. |
| Task Interval | OC: continuous telemetry ¦ annual verification |

## SS-2/SS-5 — Control Assembly

Functions in scope:

- F1: Execute brew finite state machine across 8 states

- F2: Monitor all subsystem sensors and generate fault states

- F3: Transmit maintenance status via wireless telemetry

- F4: Prevent unsafe state transitions (interlocks)

| FM-12 ¦ F1: Execute brew FSM ¦ Cat C ¦ OC |  |
| --- | --- |
| Functional Failure | FF-12: FSM enters incorrect or unrecoverable state |
| Failure Mode | Firmware corruption / watchdog timer failure / power brownout during state transition |
| Local Effect | Machine stuck in mid-brew state or permanently faulted |
| System Effect | Guest service failure; machine requires manual restart or field engineer visit |
| Hidden / End Effect | None — stuck state is visible on UI |
| Severity / Occurrence | Severity: 6/10 ¦ Occurrence: 2/10 ¦ Detectability (unaided): 2/10 |
| Consequence Category | Category C — Operational — Evident |
| Task Type | OC — On-Condition |
| Recommended Task | On-condition: watchdog timer monitoring via telemetry — alert on unexpected reset or state timeout. Firmware version checked at each maintenance visit; update to current approved build. No scheduled restoration justified — COTS microcontroller with watchdog recovery. |
| Task Interval | OC: continuous ¦ firmware check: annual |

| FM-13 ¦ F2: Monitor sensors and generate fault states ¦ Cat A ¦ FF |  |
| --- | --- |
| Functional Failure | FF-13: Sensor monitoring fails silently — fault condition not detected |
| Failure Mode | Sensor wire break / ADC failure / firmware polling error — machine continues operating without protection |
| Local Effect | Protective fault state not entered despite hazardous condition (e.g. overtemperature, empty store) |
| System Effect | Loss of multiple safety interlocks simultaneously; potential for uncontrolled hazardous conditions (burn, dry-fire, food safety breach) |
| Hidden / End Effect | CRITICAL: this failure mode is entirely hidden — machine appears normal while protection is absent |
| Severity / Occurrence | Severity: 9/10 ¦ Occurrence: 2/10 ¦ Detectability (unaided): 1/10 |
| Consequence Category | Category A — Safety / Environmental — Hidden |
| Task Type | FF — Failure Finding |
| Recommended Task | Failure-finding (mandatory): test all sensor channels at every maintenance visit — inject known out-of-range values and confirm correct fault state entry. Inspect all sensor wiring and connectors for corrosion or damage. Watchdog timer test. This is the highest-priority maintenance task in the schedule. |
| Task Interval | FF: annual (mandatory — safety-critical) |

| FM-14 ¦ F3: Wireless maintenance telemetry ¦ Cat A ¦ FF |  |
| --- | --- |
| Functional Failure | FF-14: Telemetry transmission fails — no maintenance alerts reaching cloud ops |
| Failure Mode | Wireless module failure / network configuration change / firmware error in comms stack |
| Local Effect | Maintenance alerts not transmitted; cloud ops unaware of machine faults |
| System Effect | Reactive rather than predictive maintenance; field engineer dispatched only on guest complaint; increased MTTR; potential HACCP non-compliance if milk/temperature fault goes undetected |
| Hidden / End Effect | Hidden — telemetry failure is not indicated to the hotel guest |
| Severity / Occurrence | Severity: 7/10 ¦ Occurrence: 3/10 ¦ Detectability (unaided): 2/10 |
| Consequence Category | Category A — Safety / Environmental — Hidden |
| Task Type | FF — Failure Finding |
| Recommended Task | Failure-finding: cloud ops dashboard includes heartbeat monitoring — alert generated if no telemetry received from a machine for >2 hours. Field engineer checks telemetry connectivity at each maintenance visit. Wireless module replaced if confirmed faulty. |
| Task Interval | FF: continuous heartbeat monitoring (automated) ¦ check: annual |

| FM-15 ¦ F4: Safety interlocks ¦ Cat A ¦ FF |  |
| --- | --- |
| Functional Failure | FF-15: Interlock logic fails to prevent unsafe state transition |
| Failure Mode | Firmware logic error / race condition / corrupted interlock table in memory |
| Local Effect | Unsafe operation proceeds — e.g. brew initiates with empty store, or heating continues above cutoff |
| System Effect | Depending on which interlock fails: dry-fire (FM-07), overtemperature (FM-02), or overpressure (FM-04) consequences apply |
| Hidden / End Effect | Hidden — only revealed when the condition the interlock is designed to prevent actually occurs |
| Severity / Occurrence | Severity: 10/10 ¦ Occurrence: 1/10 ¦ Detectability (unaided): 1/10 |
| Consequence Category | Category A — Safety / Environmental — Hidden |
| Task Type | FF — Failure Finding |
| Recommended Task | Failure-finding: test all interlock functions at every maintenance visit — structured test procedure per COFFCO-TC-008 and equivalent tests for thermal and pressure interlocks. Firmware integrity check (checksum verification) at each visit. |
| Task Interval | FF: annual (mandatory — safety-critical) |

## SS-1 — User Interface Panel

Functions in scope:

- F1: Accept coffee type and volume selection from hotel guest without instruction

- F2: Display machine status unambiguously (standby, brewing, fault, low consumable, empty)

| FM-16 ¦ F1: Accept guest selection ¦ Cat C ¦ OC |  |
| --- | --- |
| Functional Failure | FF-16: UI fails to accept or register user input |
| Failure Mode | Button/touchpad failure / connector fault / firmware UI handler crash |
| Local Effect | Guest cannot initiate brew; machine non-functional for user |
| System Effect | Guest service failure; complaint to hotel; housekeeping call |
| Hidden / End Effect | None — guest immediately aware |
| Severity / Occurrence | Severity: 5/10 ¦ Occurrence: 3/10 ¦ Detectability (unaided): 1/10 |
| Consequence Category | Category C — Operational — Evident |
| Task Type | OC — On-Condition |
| Recommended Task | On-condition: UI input events logged via telemetry — alert if no input registered across >3 consecutive guest room occupancies. Inspect UI panel for physical damage, liquid ingress, and button response at each maintenance visit. |
| Task Interval | OC: telemetry monitoring ¦ annual inspection |

| FM-17 ¦ F2: Display machine status ¦ Cat A ¦ FF |  |
| --- | --- |
| Functional Failure | FF-17: Status indication incorrect or absent |
| Failure Mode | LED/display failure / firmware display handler error — machine state not correctly communicated to user |
| Local Effect | Guest cannot determine machine state; may attempt to use faulted or empty machine |
| System Effect | If fault/empty state not shown: guest may repeatedly attempt brew on faulted machine; increased wear; HACCP risk if empty milk state not shown |
| Hidden / End Effect | Incorrect status display for FAULT state is a hidden failure — machine appears available when it is not safe to use |
| Severity / Occurrence | Severity: 6/10 ¦ Occurrence: 2/10 ¦ Detectability (unaided): 3/10 |
| Consequence Category | Category A — Safety / Environmental — Hidden |
| Task Type | FF — Failure Finding |
| Recommended Task | Failure-finding: test all UI status states at each maintenance visit — trigger each machine state (standby, fault, low consumable, empty) and confirm correct display. Replace UI panel assembly if any state display fails. |
| Task Interval | FF: annual |

## SS-3 — Fluid Store

Functions in scope:

- F1: Store and supply ≥2.5L of filtered water for ≥10 consecutive brews

- F2: Store and supply consumable for ≥10 consecutive brews

- F3: Filter water to tolerate 0–400 mg/L CaCO₃ without user adjustment

- F4: Contain and collect drip waste with no floor spillage

| FM-18 ¦ F1: Store and supply water ¦ Cat C ¦ OC |  |
| --- | --- |
| Functional Failure | FF-18: Water reservoir depleted before 10th brew |
| Failure Mode | Reservoir capacity insufficient / sensor calibration error reporting false 'sufficient' level |
| Local Effect | Brew fails mid-cycle or machine enters Empty state before 10 brews |
| System Effect | Guest service failure; machine unavailable until housekeeping restocks |
| Hidden / End Effect | None — empty state indicated on UI |
| Severity / Occurrence | Severity: 4/10 ¦ Occurrence: 4/10 ¦ Detectability (unaided): 2/10 |
| Consequence Category | Category C — Operational — Evident |
| Task Type | OC — On-Condition |
| Recommended Task | On-condition: water level sensor monitored continuously. Low-level alert to cloud ops when reservoir at 20% capacity. Housekeeping refill protocol triggered. Calibrate level sensor annually. |
| Task Interval | OC: continuous ¦ calibration: annual |

| FM-19 ¦ F3: Filter water / tolerate water hardness ¦ Cat D ¦ SR |  |
| --- | --- |
| Functional Failure | FF-19: Filter capacity exhausted — scale accumulates in thermal assembly |
| Failure Mode | Filter media saturation — calcium carbonate bypass into thermal system; accelerated scale build-up |
| Local Effect | Scale accumulation in heating element and pump; reduced pressure and flow |
| System Effect | Progressive degradation of FM-03 (pressure), FM-01 (temperature); shortened component life; increased MTTR on failure |
| Hidden / End Effect | Filter saturation is hidden — no direct indication to user or cloud ops unless flow/pressure monitored |
| Severity / Occurrence | Severity: 6/10 ¦ Occurrence: 6/10 ¦ Detectability (unaided): 6/10 |
| Consequence Category | Category D — Non-Operational — Evident |
| Task Type | SR — Scheduled Restoration |
| Recommended Task | Scheduled restoration: replace filter cartridge every 90 days (primary scale prevention maintenance task, consistent with 3-month descale cycle). On-condition: water hardness sensor (if fitted) triggers alert when input hardness >300 mg/L CaCO₃ for accelerated replacement schedule. |
| Task Interval | SR: 90 days ¦ OC: hardness sensor alert |

| FM-20 ¦ F4: Contain drip waste ¦ Cat C ¦ FF |  |
| --- | --- |
| Functional Failure | FF-20: Drip tray overflow — liquid reaches surface below machine |
| Failure Mode | Housekeeping failure to empty tray / tray capacity insufficient for duty cycle / tray sensor failure |
| Local Effect | Liquid on hotel room surface; potential damage to furniture; slip hazard |
| System Effect | Guest complaint; hotel liability; potential electrical hazard if liquid reaches PSU |
| Hidden / End Effect | Tray sensor failure is hidden — tray may be full without cloud ops alert |
| Severity / Occurrence | Severity: 5/10 ¦ Occurrence: 4/10 ¦ Detectability (unaided): 4/10 |
| Consequence Category | Category C — Operational — Evident |
| Task Type | FF — Failure Finding |
| Recommended Task | On-condition: drip tray level sensor monitored; alert to cloud ops and UI status indication when tray at 80% capacity. Housekeeping empties tray at each daily restocking visit. Failure-finding: test tray sensor at each maintenance visit. |
| Task Interval | OC: continuous ¦ FF: annual ¦ housekeeping: daily |

## SS-6 — Structural Chassis

Functions in scope:

- F1: Maintain footprint ≤300×300mm and mass ≤10kg throughout service life

- F2: Prevent removal of machine without tools in <60 seconds (theft deterrence)

- F3: Provide safe enclosure — no accessible live parts, no burn hazard on surface

| FM-21 ¦ F2: Theft deterrence ¦ Cat D ¦ OC |  |
| --- | --- |
| Functional Failure | FF-21: Anti-theft mechanism fails — machine removable without tools in <60s |
| Failure Mode | Anti-theft anchor fatigue / clip wear / anchor point failure in hotel furniture |
| Local Effect | Machine can be removed quickly; theft risk |
| System Effect | Machine loss; revenue impact; replacement cost; hotel reputation |
| Hidden / End Effect | Clip/anchor wear is partially hidden — only revealed on attempted removal test |
| Severity / Occurrence | Severity: 4/10 ¦ Occurrence: 3/10 ¦ Detectability (unaided): 5/10 |
| Consequence Category | Category D — Non-Operational — Evident |
| Task Type | OC — On-Condition |
| Recommended Task | On-condition: inspect anti-theft anchor and clip mechanism at each maintenance visit — apply firm pull test (>50N) and confirm <2mm play. Replace anchor assembly if any play detected. Inspect hotel furniture anchor point for damage. |
| Task Interval | OC: annual inspection with pull test |

| FM-22 ¦ F3: Safe enclosure — no accessible live parts, no burn surface ¦ Cat B ¦ OC |  |
| --- | --- |
| Functional Failure | FF-22: Casing damage creates accessible burn surface or live-part exposure |
| Failure Mode | Physical impact damage to casing / guest or housekeeping mishandling / manufacturing defect |
| Local Effect | Exposed hot surface or live part accessible to hotel guest or housekeeping |
| System Effect | Burns or electrocution risk; regulatory non-compliance; hotel liability; potential product recall |
| Hidden / End Effect | Internal damage from impact may not be visible externally — burn/shock hazard hidden |
| Severity / Occurrence | Severity: 9/10 ¦ Occurrence: 2/10 ¦ Detectability (unaided): 4/10 |
| Consequence Category | Category B — Safety / Environmental — Evident |
| Task Type | OC — On-Condition |
| Recommended Task | On-condition: housekeeping staff instructed to report any visible casing damage at each daily check. Machine taken out of service immediately on any casing damage report. Formal casing inspection at each maintenance visit — inspect all panels, edges, and ventilation openings. Replace casing panels on any damage. |
| Task Interval | OC: daily visual (housekeeping) ¦ formal: annual |

# 4.0  Recommended Maintenance Programme

The following scheduled maintenance programme is derived directly from the RCM2 task selection above. Tasks are grouped by maintenance level and interval.

## 4.1  Daily — Housekeeping (Non-Technical)

| FM | SS | Task | Type |
| --- | --- | --- | --- |
| FM-09 | SS-4 | Visual inspection of casing for physical damage, liquid ingress, or exposed components. Report any damage immediately; machine taken out of service. | OC/FF |
| FM-20 | SS-3 | Empty drip tray if 80% full indicator active. Visual check for overflow. | OC |
| FM-06 | SS-2/3 | Confirm milk store temperature indicator shows normal (≤4°C). Report if fault indicated. | OC |
| FM-22 | SS-6 | Visual inspection of casing panels for impact damage. Report any crack or deformation. | OC |

## 4.2  90-Day — Field Engineer (Scheduled Restoration)

| FM | SS | Task | Type |
| --- | --- | --- | --- |
| FM-03 | SS-2/3 | Descale programme: run full descale cycle per approved procedure. Record pre/post pressure measurements. | SR |
| FM-19 | SS-3 | Replace water filter cartridge. Record lot number and replacement date. | SR |

## 4.3  Annual — Field Engineer (Full Maintenance Visit)

All tasks below are mandatory at each annual maintenance visit. Safety-critical failure-finding tasks (Category A) cannot be deferred.

| FM | SS | Task | Cat | Type |
| --- | --- | --- | --- | --- |
| FM-07 | SS-2/3 | Dry-fire interlock test (COFFCO-TC-008): drain fluid store, command brew, confirm no heating element activation, fault state within 5s. MANDATORY SAFETY TASK. | A | FF |
| FM-09 | SS-4 | Hipot insulation resistance test per IEC 60335. Record result. Machine out of service if fail. | A | FF |
| FM-13 | SS-2/5 | Test all sensor channels — inject out-of-range values; confirm correct fault state entry for each. Record results for all channels. | A | FF |
| FM-14 | SS-2/5 | Verify telemetry connectivity end-to-end: trigger test alert; confirm receipt at cloud ops dashboard within 60s. | A | FF |
| FM-15 | SS-2/5 | Test all interlock functions: thermal, pressure, dry-fire, and empty-store interlocks. Firmware checksum verification. | A | FF |
| FM-17 | SS-1 | Test all UI status displays: trigger each machine state and confirm correct display. Replace UI panel if any state display fails. | A | FF |
| FM-02 | SS-2/3 | Test secondary thermal cutoff: confirm it activates independently of primary control if temperature >95°C. | B | FF |
| FM-01 | SS-2/3 | Inspect heating element; clean scale if not addressed by 90-day descale. Record outlet temperature measurement. | C | OC |
| FM-04 | SS-2/3 | Inspect and test pressure relief valve function. Replace if any restriction or sluggish operation detected. | B | OC |
| FM-05 | SS-2/3 | Inspect frother nozzle; clean if not done at 30-cycle interval. Calibrate milk temperature sensor. | C | OC |
| FM-10 | SS-4 | Inspect plug module clip mechanism; replace if wear or play detected. | C | OC |
| FM-11 | SS-4 | Measure standby power with calibrated power meter. Confirm ≤0.5W. Alert CoffCo engineering if exceeded. | D | OC |
| FM-12 | SS-2/5 | Check firmware version; update to current approved build. Review watchdog reset log since last visit. | C | OC |
| FM-16 | SS-1 | Test all UI input controls for correct registration. Inspect for physical damage and liquid ingress. | C | OC |
| FM-18 | SS-3 | Calibrate water level sensor against known fill volume. | C | OC |
| FM-20 | SS-3 | Test drip tray level sensor function: fill to 80%, confirm alert generated. | C | FF |
| FM-21 | SS-6 | Anti-theft anchor pull test (>50N). Inspect clip and anchor point. Replace if >2mm play. | D | OC |
| FM-22 | SS-6 | Full casing inspection: all panels, edges, ventilation openings. Replace any damaged panels. | B | OC |

## 4.4  Scheduled Discard Items

| FM | SS | Item | Interval | Basis |
| --- | --- | --- | --- | --- |
| FM-02 | SS-2/3 | Secondary thermal cutoff | 3 years | Safe life; hidden failure risk |
| FM-04 | SS-2/3 | Pressure relief valve | 3 years | Safe life; safety consequence |
| FM-10 | SS-4 | Regional plug module | 5 years | Wear mechanism; arcing risk |

# 5.0  Study Summary and Conclusions

| Metric | Result |
| --- | --- |
| Total failure modes analysed | 25 across 6 subsystems |
| Category A — Safety/Environmental Hidden | 6 (FM-07, FM-09, FM-13, FM-14, FM-15, FM-17) |
| Category B — Safety/Environmental Evident | 4 (FM-02, FM-04, FM-06, FM-22) |
| Category C — Operational | 9 |
| Category D — Non-Operational | 3 |
| On-Condition (OC) tasks | 12 |
| Failure-Finding (FF) tasks | 7 (6 mandatory safety-critical) |
| Scheduled Restoration (SR) tasks | 2 (90-day descale + filter) |
| Scheduled Discard (SD) items | 3 (thermal cutoff, relief valve, plug module) |
| Run to Failure (RTF) | 1 (FM-08: PSU module — immediately evident) |
| Primary maintenance driver | Scale accumulation (FM-03, FM-19) — 90-day descale cycle |
| Highest-risk failure mode | FM-07 / FM-13 / FM-15 — hidden safety interlocks (Sev 9–10, hidden) |
| Annual maintenance visit justified? | YES — 18 tasks at annual interval; 6 mandatory FF safety tasks |
| MTBF target (≥26,280 hours) achievable? | YES — subject to 90-day descale and annual maintenance compliance |

Key conclusions from this RCM2 study:

- Scale accumulation is confirmed as the primary maintenance driver. The 90-day descale and filter replacement cycle is the single most important scheduled task for operational availability.

- Six Category A (hidden safety) failure modes exist, all requiring mandatory annual failure-finding tasks. These cannot be deferred without formal SDA risk acceptance. FM-07 (dry-fire interlock) and FM-13 (sensor monitoring failure) are the highest-priority tasks.

- The cloud telemetry system (SS-2/SS-5) is central to the maintenance strategy — 10 of 22 failure modes rely on continuous telemetry monitoring as the primary on-condition detection mechanism. TD-06 (wireless technology selection) resolution is therefore a maintenance programme dependency.

- One run-to-failure decision is justified: FM-08 (PSU module failure). Failure is immediately evident and COTS replacement is the most cost-effective response.

- The recommended annual maintenance visit covers all 18 annual tasks within a single field engineer visit (estimated 4 hours), consistent with the ≥365-day maintenance interval target in SRS 3.3.

| Systems Design Authority: _________________________________ Date: __________ | Lead Systems Engineer: _________________________________ Date: __________ |
| --- | --- |