Integrated Product Support Plan

CoffCo Coffee Machine

Structured against the ILS framework — all 10 ILS branches addressed

Evidence base: Fan Jinjuan et al. (2009) Engineering Failure Analysis; COFFCO-HAZID-001 v2.0; COFFCO-FMECA-001

| Field | Detail |
| --- | --- |
| Document Reference | Integrated Product Support Plan CoffCo Coffee Machine V3.0 |
| Issue | 3.0 |
| Date | June 2026 |
| Prepared by | CoffCo Systems Team / School of Systems Engineering Secondment |
| Approved by | Pending — Systems Design Authority (SDA) |
| ILS Framework | Structured against 10 ILS branches: Maintenance Support ¦ Computer Support ¦ Facilities ¦ PHS&T ¦ Support & Test Equipment ¦ Technical Data ¦ Training ¦ Personnel ¦ Supply Support ¦ Engineering Support |
| System Life | 5–7 years per unit at 2 brews/day |
| Target Availability | Ao ≥ 98.5% (≥40 weeks/year fully available per room) |

# Document History

| Issue | Date | Author | Description | Approved |
| --- | --- | --- | --- | --- |
| 1.0 | June 2026 | CoffCo / SSE | Initial issue. ILS plan based on COFFCO-HAZID-001, FMECA, and SRS. Three-tier maintenance concept, MTBF/MTTR targets, spares list. | Pending |
| 2.0 | June 2026 | CoffCo / SSE | Iterative review. External failure analysis data (Fan Jinjuan et al., 2009) integrated. Seven top-failure components identified. Maintenance schedule PM-01 to PM-07, SRL, diagnostic equipment, IPS risks added. | Pending |
| 3.0 | June 2026 | CoffCo / SSE | Iterative review against ILS framework (all 10 branches). Added: Computer Support (hardware, software, cybersecurity, firmware); Facilities (depot, training, storage, utilities); PHS&T (packaging, handling, storage, transportation); Technical Data (drawings, data management, configuration); Personnel (workforce planning, human factors, recruitment); Supply Support (rotables, obsolescence management, procurement). Full coverage across all ILS branches. | Pending |

# ILS Branch Coverage — Summary Map

The following table maps every branch and sub-element of the ILS framework to the relevant section of this plan. This provides a single navigational reference confirming all 10 ILS branches are addressed.

| ILS Branch | Key Sub-Elements | CoffCo IPS Response | Section |
| --- | --- | --- | --- |
| 1. Maintenance Support | Preventive/corrective; 3 tiers; repair levels; task analysis; BITE; manpower | Three-tier concept; PM-01 to PM-07 schedule; FIDA task analysis; corrective response matrix; manpower table | §1 |
| 2. Computer Support | Servers/workstations/UPS; software; databases; cybersecurity; firmware | CoffCo cloud platform architecture; maintenance management system; data protection; OTA firmware; penetration testing | §2 |
| 3. Facilities | Workshop; storage; training facilities; utilities; garaging | CoffCo regional depot specification; assembly storage conditions; field engineer training centre; utilities | §3 |
| 4. PHS&T | Protective packaging; hazmat; manual handling; inventory storage; shipping; regulatory | Assembly packaging specification; WEEE/RoHS compliance; manual handling; climate storage; carrier compliance | §4 |
| 5. Support & Test Equipment | Hand tools; diagnostic devices; calibration; simulation rigs; mobile workstations | Field engineer toolkit; calibrated instruments; CoffCo diagnostic tablet; training rig specification | §5 |
| 6. Technical Data | Manuals; schematics; CAD; specifications; version control; configuration | Online manual strategy; CAD/drawing control; SRS/standards reference; data management | §6 |
| 7. Training | TNA; operator/maintainer training; e-learning; simulators; facilities | TNA (3 stakeholders); delivery methods; e-learning; training rig; qualification programme | §7 |
| 8. Personnel | Workforce planning; skill requirements; human factors; recruitment/retention | Headcount by tier; certification requirements; ergonomics; career framework | §8 |
| 9. Supply Support | Provisioning; inventory; rotables; procurement; obsolescence | SRL; rotable cycle; reorder points; vendor strategy; approved equivalent list; lifecycle tracking | §9 |
| 10. Engineering Support | Supportability; config management; field data; engineering change | Design for maintainability; ECN process; field data feedback (Fan et al.); Fleet analytics | §10 |

# §1. Maintenance Support

| 1 Maintenance Support | Preventive vs corrective ¦ Scheduled vs unscheduled ¦ 3 Tiers ¦ Repair/Replace ¦ Task Analysis ¦ BITE ¦ Manpower |
| --- | --- |

## 1.1 Target Availability and RAM Metrics

| Metric | Target | Rationale | SRS Ref |
| --- | --- | --- | --- |
| Operational Availability (Ao) | ≥ 98.5% | 358 available days/year per room minimum. 1.5% downtime budget allocated across corrective, preventive, and logistics delay. | SRS 3.3, 3.9 |
| MTBF | ≥ 26,280 h | 3× annual powered hours. System-level reliability allocation. | SRS 3.9 |
| MTTR | ≤ 4 hours | Assembly-swap corrective maintenance. Tier 2 field engineer. | SE Proposal ILS |
| Mean Down Time (MDT) | ≤ 8 hours | MTTR + logistics delay (depot stock retrieval). | Derived |
| Preventive Maintenance Interval | ≥ 365 days | No scheduled maintenance more than once per year (housekeeping daily service excluded). | SRS 3.3 |

## 1.2 Three-Tier Maintenance Concept

| Tier | Who | Scope (Preventive + Corrective) | Trigger |
| --- | --- | --- | --- |
| Tier 1 Organisational | VS Housekeeping (no tools) | PM-01: Daily consumable, drip tray, cleaning, visual check, HACCP milk, status report. PM-02: monitor firmware descale cycle. No repair scope. | Daily — every room service visit |
| Tier 2 Intermediate | CoffCo Field Engineer | PM-03 annual (seal replace, PRV test, pressure/temp verify, NTC inspect, firmware update). PM-04 NTC holder inspect. PM-05 PRV test. Corrective: assembly swap (Thermal, Control, PSU). Diagnostic: fault code read, FIDA procedures. | Alert-triggered (cloud ops) or annual scheduled |
| Tier 3 Depot / Engineering | CoffCo Engineering | PM-06 fleet telemetry review (quarterly). Assembly refurbishment after swap-and-return. Design-level investigation. PM-07 end-of-life assessment. | Escalated from Tier 2; quarterly fleet review; Year 5 EOL |

## 1.3 Maintenance Schedule — PM-01 to PM-07

| ID | Task | Trigger | Procedure Summary | Tier | Down-time |
| --- | --- | --- | --- | --- | --- |
| PM-01 | Daily check | Daily | Restock consumables (<3 remaining). Empty drip tray. Wipe exterior. Check status indicator. Report FAULT/MAINTENANCE REQUIRED. | Tier 1 | 0 |
| PM-02 | Descale cycle | Brew count / DESCALE-REQUIRED alert | Automated firmware cycle (~20 min). Housekeeping ensures reservoir filled. Confirm STANDBY after completion. Log via cloud ops. | Tier 1 + Auto | 20 min |
| PM-03 | Annual preventive | 12 months from commissioning | Full inspection: seal replacement (all FKM O-rings); pump strainer clean; PRV test; TCO continuity; WDT IC test; firmware update; HACCP fluid path clean; extraction pressure and temperature verify; as-built config record update. | Tier 2 | 2–3 h |
| PM-04 | NTC holder inspect | Annual at PM-03 or after SAFE-SHUTDOWN | Visual: check for discolouration, distortion, perforation (Fan et al. simulation evidence). Replace on any deformation. | Tier 2 | Incl. in PM-03 |
| PM-05 | PRV test | Annual at PM-03 or after SAFE-SHUTDOWN | Bench pump: verify opens ≤2.5 bar; non-venting ≤1.3 bar; fluid to drip tray only. Replace if out of spec. | Tier 2 | Incl. in PM-03 |
| PM-06 | Fleet telemetry review | Quarterly | Cloud ops: SAFE-SHUTDOWN frequency; descale intervals vs hardness; MILK-SPOILAGE-RISK rate; COMMS-FAULT rate. Flag properties with anomalous fault rates. | Tier 3 | Remote |
| PM-07 | End-of-life assessment | Year 5 | Field engineer: (1) Extraction pressure at outlet vs. spec. (2) Fluid path seal condition - visual inspect for weeping. (3) Chassis integrity - casing, PPS lid fixings, ventilation slots. (4) Firmware supportability - confirm OTA update support continues. (5) Electrical integrity check (Fan et al. evidence - insulating layer aging and open circuit of wires are basic events in the electricity leakage FTA): insulation resistance test on PSU primary-to-secondary and primary-to-earth (pass criterion: greater than 1 MOhm per IEC 60335-1); visual inspection of mains wiring for insulation discolouration, cracking, or chafing; check solder joints and crimp connections at PSU terminals for fatigue cracking. If insulation resistance below 1 MOhm or visible wiring damage found: mandatory PSU swap before machine returns to service. Confirm COTS PSU supplier stated design life has not been exceeded - if design life is 5 years or less, treat as a hard swap trigger. Decision: extend service life, refurbish, or replace. | Tier 2/3 | 1 h/machine |

## 1.4 Manpower and Skills

| Role | Quantity | Employment Type | Key Competencies | Security Clearance |
| --- | --- | --- | --- | --- |
| VS Housekeeping | Per hotel property — existing headcount | VS civilian staff | CoffCo 45-min onboarding; damp-cloth cleaning; status indicator literacy; HACCP milk awareness | No clearance required |
| VS Maintenance Team | 1–2 per property — existing hotel maintenance staff | VS civilian staff | Basic M&E maintenance; FIDA fault procedures; safe power isolation; fault escalation | No clearance required |
| CoffCo Field Engineer | Minimum 2 per region (scaling with estate) | CoffCo employed or contracted | 2-day Field Service qualification; assembly swap; calibrated instruments; HACCP; diagnostic tablet; IEC 60335 awareness | Basic DBS check — hotel site access |
| CoffCo Cloud Ops | 2–4 FTE (24/7 rotation) | CoffCo employed | Cloud platform; telemetry triage; OTA firmware; FIDA escalation decisions | Cyber access control — cloud platform credentials |
| CoffCo Field Engineer — Senior (Tier 3 escalation) | 1–2 per region | CoffCo employed | All Tier 2 competencies plus design-level fault investigation; firmware fault analysis; fleet analytics | Standard DBS + CoffCo engineering NDA |

# §2. Computer Support

| 2 Computer Support | Hardware ¦ Software ¦ Databases ¦ Cybersecurity ¦ Firmware |
| --- | --- |

## 2.1 Hardware

| Hardware Element | Specification | Role in CoffCo IPS |
| --- | --- | --- |
| Cloud platform servers | Hosted cloud infrastructure (AWS / Azure / GCP — TBD at TD-06 resolution). Minimum 99.9% uptime SLA. Auto-scaling for fleet telemetry load. | Receives all machine telemetry; hosts maintenance alerts; OTA firmware distribution; as-built config register for all deployed machines. |
| CoffCo cloud ops workstations | Standard enterprise spec. Dual monitor. Secure remote access VPN for out-of-hours triage. | Fleet monitoring dashboard; alert triage; OTA management; fleet analytics (PM-06). |
| CoffCo diagnostic tablets (field) | Ruggedised Android or iOS tablet. Pre-loaded with Field Service Manual, FIDA procedures, diagnostic dongle driver, and offline sync of machine config records. | Used by Tier 2 field engineers at every service visit. Works offline — syncs to cloud when Wi-Fi available. |
| UPS (cloud ops) | Uninterruptible power supply for cloud ops workstation cluster. Minimum 30 min autonomy. | Ensures cloud ops can continue during hotel power outages. Critical for 24/7 monitoring model. |
| VS maintenance tablet | Standard tablet (VS-provided). Browser-based access to FIDA Maintenance Manual (web-hosted). | Vacation Stables maintenance team access to FIDA procedures in the field without specialist hardware. |

## 2.2 Software and Data Systems

| System | Function | Data Held |
| --- | --- | --- |
| CoffCo Cloud Platform | Central system: telemetry ingestion, alert generation, OTA firmware, fleet dashboard, machine registration, as-built config records, commissioning records. | Machine serial numbers; firmware versions; water hardness calibrations; fault history per machine; consumable level trends; SAFE-SHUTDOWN events. |
| Maintenance Management System (MMS) | Tracks all Tier 2 field engineer callouts and annual service visits. Generates work orders. Records PM completion and parts used. Links to SRL for stock depletion tracking. | Open/closed work orders; PM-03 to PM-07 records; assembly swap history per machine; spare parts consumption. |
| Fleet Analytics Platform | Quarterly PM-06 analysis: fault frequency by machine, property, and region. Predictive maintenance models (scale accumulation rate vs descale interval). Benchmarks performance against Fan et al. failure mode frequencies. | Aggregate fleet telemetry; fault trend analysis; MTBF estimates per assembly type; descale interval calibration data by property. |
| Technical Publications CMS | Hosts all online manuals (User Manual, Housekeeping Guide, FIDA Maintenance Manual, Field Service Manual). Version-controlled. QR code links always resolve to current version. | All manual versions with change history; version numbers; effective dates; linked SRS requirement references. |
| Spares Inventory System | Tracks depot stock levels per region. Triggers reorder at minimum stock threshold. Manages rotable cycle (swap-out assembly status: in-service, returned, under repair, refurbished, available). | Stock counts by part number and region; reorder points; lead times; rotable status. |

## 2.3 Cybersecurity

The CoffCo cloud platform connects IoT devices in hotel rooms to a central cloud infrastructure. This creates an attack surface that must be addressed in the IPS plan.

| Cybersecurity Element | Control | Rationale |
| --- | --- | --- |
| Access control | Role-based access control (RBAC) on cloud platform. CoffCo cloud ops: full access. VS property managers: read-only fleet view for their property. Field engineers: machine-level config access for registered machines only. | Limits blast radius of credential compromise. VS cannot access other customers' data. |
| Data protection | All telemetry data encrypted in transit (TLS 1.3) and at rest (AES-256). Machine telemetry contains operational data only — no guest personal data (GDPR Article 6(1)(f)). Data processing agreement signed with Vacation Stables before go-live. | GDPR compliance. Hotel room IoT is a sensitive deployment context. Guest data must never be collected. |
| OTA firmware security | Firmware updates signed with CoffCo private key. Machine verifies signature before installing. Rollback capability retained. Update window restricted to 03:00–06:00 local time. | Prevents malicious firmware injection. Signed updates ensure only authorised CoffCo firmware can be installed. |
| Penetration testing | Annual penetration test of cloud platform API by accredited third-party security firm. Results reviewed by CoffCo engineering. Critical findings remediated before next estate rollout wave. | Connected IoT in hotel rooms is a target for network intrusion. Annual pen testing confirms security posture. |
| Network segmentation | CoffCo devices operate on a dedicated IoT VLAN (or equivalent) on the Vacation Stables hotel network — isolated from guest and administrative traffic. Agreed with VS IT before go-live. | Prevents CoffCo machines from being used as an entry point to the hotel network. |
| Diagnostic tablet security | CoffCo diagnostic tablets are managed devices (MDM enrolled). Remote wipe capability. Encrypted storage. PIN/biometric lock. Cannot be used for personal internet access. | Tablets contain machine config data and Field Service Manual. If lost, data must be recoverable and wipe-capable. |

## 2.4 Firmware Management

- Firmware version controlled in CoffCo Git repository. Each deployed machine has a current firmware version recorded in the cloud platform asset register.

- OTA updates pushed by cloud ops during low-usage window (03:00–06:00 local time). Machine must be in STANDBY state. Update process: download → signature verify → install → reboot → confirm STANDBY. Rollback triggered automatically if machine does not reach STANDBY within 120 s post-update.

- Manual firmware update via diagnostic tablet used when machine has no Wi-Fi access (fallback for TD-06 failure or hotel IT issues).

- Firmware change notes captured in Technical Publications CMS and linked to relevant SRS requirement. If update changes an operating procedure, associated online manual updated before firmware is deployed.

# §3. Facilities

| 3 Facilities | Maintenance workshop ¦ Storage ¦ Training ¦ Utilities ¦ Garaging |
| --- | --- |

## 3.1 CoffCo Regional Depot — Maintenance and Storage Facility

| Facility Element | Specification | Rationale |
| --- | --- | --- |
| Workshop | Minimum 30 m² workshop per regional depot. Assembly-swap workbench (2×). ESD (electrostatic discharge) protected workstations for Control Assembly handling. Adequate lighting. Ventilation for HACCP cleaning agents. | Tier 3 assembly refurbishment and testing. ESD protection required for Control Assembly PCBs. Ventilation required for food-grade cleaning agents used after Thermal Assembly swap. |
| Assembly storage | Racking for returned assemblies (awaiting repair) and refurbished assemblies (available for dispatch). Climate-controlled: 10–25°C, <60% relative humidity. Food-grade storage for Thermal Assembly (HACCP requirement). | Scale deposits and seal materials degrade with temperature cycling. Thermal Assembly food-grade storage required — HACCP fluid path must not be contaminated during storage. |
| Spares storage | Separate racking for consumable spares (seal kits, PRVs, NTC holders, fuses, descale fluid). Inventory system terminal at racking. First-in-first-out (FIFO) rotation enforced for items with shelf life. | Seal kits and descale fluid have finite shelf life. FIFO rotation prevents expired stock reaching field engineers. Inventory system integration enables automated reorder point monitoring. |
| Consumable refrigeration | Refrigerated storage for milk-path-related spares where applicable (HACCP-controlled). Temperature log maintained. | HACCP compliance for any components that contact the milk fluid path. |
| Access control | Keypad + audit log access to assembly storage area. Authorised persons list maintained by CoffCo depot manager. No access for VS staff or contractors without CoffCo escort. | Assemblies contain CoffCo IP. Access control protects against theft or unauthorised modification. Audit log supports traceability if a recalled assembly is dispatched in error. |
| Utilities | Three-phase power supply (for test equipment). Cold water (for Thermal Assembly post-refurbishment test brew). Standard IT connectivity (Ethernet + Wi-Fi for diagnostic tablets and MMS terminal). | Test brew requires water supply at depot. Three-phase power for calibration rigs. IT connectivity for MMS and cloud platform access. |

## 3.2 Field Engineer Garaging and Mobile Facilities

- Field engineers operate from home or regional depot — no dedicated field office required. Service vehicles carry the full field engineer service kit (see §5).

- Service vehicle specification: minimum estate car or small van. Internal racking for assembly transit cases. Power inverter for diagnostic tablet charging. Refrigerated cooler bag for HACCP-critical components if Thermal Assembly transit requires temperature control.

- Hotel rooms provide the working environment for Tier 2 maintenance. No special facilities required at hotel — field engineer brings all tools.

## 3.3 Training Facilities

| Facility | Purpose and Specification |
| --- | --- |
| CoffCo Field Engineer Training Centre | Minimum 1 dedicated training centre per CoffCo operating region. Equipped with: 4–6 representative CoffCo machines (production-representative, not necessarily certified units); full field engineer service kit (1 set per 2 trainees); calibration rigs; water supply; HACCP cleaning facilities; classroom space (12 seats); projector and digital display for training materials. Used for 2-day Field Service qualification and annual refresher. |
| Classroom (at training centre) | 12-seat classroom for theory component of 2-day qualification. Whiteboard and digital display. FIDA procedure walkthroughs. Safety case review (IEC 60335 awareness). Diagnostic tool software training. |
| Practical practice areas (at training centre) | Assembly-swap practical exercises on representative machines (not guest-deployed units). Calibrated pressure gauge and thermocouple exercises. Simulated fault injection (firmware hang for WDT test; empty reservoir for dry-fire interlock; PRV bench test). HACCP cleaning procedure practice. |
| Simulators | CoffCo diagnostic tablet simulation environment: pre-loaded with simulated fault code scenarios for diagnostic training without a live machine. Used for cloud ops training and for VS maintenance team FIDA familiarisation during on-site training. |
| On-site training environment | Each hotel property at commissioning serves as an on-site training environment. The installed machine (post-commissioning sign-off) is used for Housekeeping and VS maintenance team on-site training. Delivered by field engineer at each property before go-live. |

# §4. Packaging, Handling, Storage, and Transportation (PHS&T)

| 4 PHS&T | Protective packaging ¦ Hazardous materials ¦ Manual handling ¦ Inventory storage ¦ Shipping ¦ Regulatory compliance |
| --- | --- |

## 4.1 Packaging

| Item | Packaging Specification |
| --- | --- |
| New machine (outbound) | Double-wall corrugated carton with custom-fit polyethylene foam inserts (minimum 50mm). Rated for drops up to 1.0m per ASTM D5276. Anti-tip bracket and Quick-Reference Card in separate compartment within same carton. Carton labelled with: machine serial number, destination room number, property name, and ‘FRAGILE — HOT LIQUID APPLIANCE’. |
| Assembly swap (outbound — replacement) | Transit case: IP54-rated carry case with foam inserts. Thermal Assembly: additional HACCP-compliant sealed food-grade bag within transit case. Control Assembly: ESD shielding bag within transit case. Clearly labelled: ‘REPLACEMENT ASSEMBLY — COFFCO FIELD ENGINEER USE ONLY’. |
| Assembly swap (return — failed unit) | Same transit case, returned with failed assembly. Return label pre-printed by cloud ops MMS on fault alert generation. Label includes: machine serial number, fault description, date of removal, field engineer ID. |
| Consumables (ongoing supply) | Standard protective carton. FIFO rotation enforced — carton date-stamped. Descale fluid: classified as non-hazardous cleaning agent — labelled per GHS/CLP. Seal kits: sealed poly bag within carton, desiccant included. |
| Hazardous materials | No Class 1 (explosive) or Class 8 (corrosive) materials. Descale fluid (citric acid based): GHS07 (irritant), non-hazardous for transport (UN3082 if applicable — confirm with CoffCo compliance). WEEE marking on all electronic assemblies per Directive 2012/19/EU. |

## 4.2 Handling

- Thermal Assembly: maximum mass 4 kg. One-person lift within manual handling guidelines (HSE Manual Handling Operations Regulations 1992). No mechanical aids required for routine assembly swap.

- Full machine (new installation): maximum mass 10 kg (SRS 4.10). One-person carry acceptable for flat-surface hotel room delivery. Use of sack truck for multi-floor hotel deliveries where lift access is unavailable.

- ESD precautions: anti-static wrist strap mandatory when handling Control Assembly PCB. ESD bag used during transit.

- HACCP handling: Thermal Assembly handled with food-grade disposable gloves when any fluid path surfaces are accessible. HACCP cleaning completed before assembly is returned to storage.

## 4.3 Storage (Depot and Property)

| Item | Storage Conditions | Max Storage Duration | Environmental Protection |
| --- | --- | --- | --- |
| Thermal Assembly (refurbished, awaiting dispatch) | 10–25°C, <60% RH. Climate-controlled racking. HACCP food-grade area. | 12 months maximum before re-test brew required | Sealed food-grade bag. Desiccant in transit case. |
| Control Assembly | 10–25°C, <60% RH. ESD-protected shelving. | 24 months | ESD shielding bag. Do not store near magnetic field sources. |
| PSU | Ambient (5–40°C). Dry. No special requirement. | 36 months | Standard packaging. Protect from moisture. |
| Fluid path seal kits (FKM) | 10–25°C, <65% RH. Dark storage (UV degrades FKM). | 24 months from manufacture | Sealed poly bag with desiccant. FIFO rotation enforced. |
| PRV (Pressure Relief Valve) | Dry, ambient. Spring must not be compressed in storage. | 36 months | Sealed individual poly bag. Do not stack. |
| Descale fluid | Ambient (5–35°C). Away from heat sources and direct sunlight. | 24 months from manufacture | GHS-labelled sealed container. Separate from food-contact items. |
| Hotel property consumable stock | Housekeeping store: ambient, dry, clean. Not accessible to guests. | Rotate per demand rate (2 brews/day/room). No long-term storage. | Standard packaging. First-in-first-out. |

## 4.4 Transportation

| Shipment Type | Method and Carrier | Regulatory Compliance |
| --- | --- | --- |
| New machine — estate rollout (bulk) | Palletised delivery to hotel goods entrance. Pallet wrap. Carrier: DHL Freight, XPO, or equivalent. | No IATA restrictions (mains appliance, no lithium cells). WEEE marking per EU Directive 2012/19/EU. Carton marked ‘FRAGILE’. |
| New machine — single replacement | Parcel carrier: UPS, Royal Mail, DPD. Next-day delivery. Tracked. | Same as above. Returns: completed WEEE disposal if machine at end of life. |
| Assembly swap (field engineer vehicle) | Field engineer carries replacement in transit case in service vehicle. No courier required. | Standard road transport. Food-grade Thermal Assembly in sealed HACCP bag. |
| Assembly return (failed unit to depot) | UPS or Royal Mail. Return label pre-printed by MMS on fault alert. Collected from hotel by arrangement. | WEEE labelling on electronic assemblies. Descale fluid: GHS07 labelling; confirm non-hazmat classification with CoffCo compliance. |
| International shipment (EU/US markets) | Freight forwarder with customs brokerage. CE declaration (EU), UL/ETL certificate (US) included in shipping documentation. | Import duties per destination market. RoHS compliance documentation. Country-specific plug module variant confirmed before dispatch. |

# §5. Support and Test Equipment

| 5 Support & Test Equip. | General tools ¦ Diagnostic devices ¦ Calibration ¦ Simulation rigs ¦ Maintenance aids |
| --- | --- |

| Equipment | Type | Specification / Purpose | Who Holds It |
| --- | --- | --- | --- |
| CoffCo diagnostic tablet + dongle | Specialist diagnostic | Ruggedised tablet. Pre-loaded: FIDA procedures, Field Service Manual, fault code library, firmware update tool, machine config access. Auto-sync to cloud when connected. | CoffCo field engineer (1 per FE). Spare at regional depot. |
| Calibrated pressure gauge | Calibration tool | Range 0–5 bar, accuracy ±0.05 bar. NIST-traceable calibration certificate. Annual recalibration required. | CoffCo field engineer kit. Calibration managed by CoffCo depot. |
| Calibrated thermocouple (Type K) | Calibration tool | Range 0–200°C, accuracy ±0.5°C. Annual recalibration required. Used for TCO test (TC-HE-01) and brew temp verify. | CoffCo field engineer kit. |
| Water hardness test kit | Diagnostic | Range 0–500 mg/L CaCO₂. Colorimetric or digital. Used at commissioning and if water supply changes. | CoffCo field engineer kit. Spare at regional depot. |
| Assembly swap toolset | General tools | Anti-tamper torque screwdriver (3–15 Nm). Spanner set (metric). Food-grade nitrile gloves. ESD wrist strap and mat. Hex key set. | CoffCo field engineer kit (in carry case). |
| HACCP cleaning kit | Specialist | Food-grade cleaning agent (non-tainting). Brushes and swabs for fluid path cleaning. pH test strips for rinse water verification. Disposable gloves. | CoffCo field engineer kit. Consumable — replenished from depot. |
| Seal installation tool set | Specialist | O-ring picks (non-metallic). Seal lubricant (food-grade silicone grease). Prevents installation damage identified as root cause of early seal failure (Fan et al.). | CoffCo field engineer kit. |
| Bench pump (PRV test rig, depot only) | Simulation rig | Calibrated bench pump 0–5 bar for PRV functional test (PM-05). Fixed at regional depot for depot-level PRV verification of returned assemblies. | CoffCo regional depot (fixed equipment). |
| Fault injection rig (training centre only) | Simulation rig | Allows controlled simulation of fault conditions for training: firmware hang (WDT test), empty reservoir (dry-fire), PRV test. Used during 2-day FE qualification. | CoffCo training centre (fixed equipment). |
| Mobile workstation (field engineer vehicle) | Maintenance aid | Tablet charging adapter (12V vehicle power). Portable light for hotel room work. Cable ties and labelling kit for post-swap cable management. | CoffCo field engineer kit (in service vehicle). |
| Sack truck / hand truck | Handling aid | For multi-floor hotel deliveries where lift is unavailable. Collapsible, stored in service vehicle. | CoffCo field engineer service vehicle. |

# §6. Technical Data

| 6 Technical Data | Manuals ¦ Schematics ¦ CAD ¦ Specifications ¦ Data management ¦ Version control |
| --- | --- |

## 6.1 Documentation Suite

| Document | Content | Format / Delivery | Audience | Version Control |
| --- | --- | --- | --- | --- |
| User Manual | 9-step operating sequence; visual aids; tips; troubleshooting; coffee education; multi-language. | Web-hosted (QR code). PDF download. | Hotel room guest | CoffCo CMS. Updated before any firmware change affecting operation. |
| Housekeeping Guide | Daily routine; cleaning; status indicators; HACCP milk rule; fault escalation. | Web-hosted (QR code). Physical Quick-Reference Card. | VS housekeeping | CoffCo CMS. Updated if procedure changes. |
| FIDA Maintenance Manual | Fault ID/Diagnosis/Action for all alerts and fault codes. Tier matrix. Worked examples. | Web-hosted (login-gated). Offline PDF on VS maintenance tablet. | VS maintenance team, CoffCo field engineers | CoffCo CMS. Signed by SDA before publication. |
| Field Service Manual | Full assembly-swap procedures; safety interlock testing; firmware update; HACCP cleaning; diagnostic tool use; fault code library. | Pre-loaded on diagnostic tablet. Offline-capable. | CoffCo field engineers | CoffCo CMS + tablet MDM. Field engineers notified of updates. |
| HACCP Documentation | Food safety plan; approved cleaning products; temperature monitoring; milk handling. | Controlled document (PDF). Lodged with VS food safety officer. | VS food safety, CoffCo engineering | SDA-signed. Annual review. |
| IEC 60335-1 Certificate | Type-test certificate confirming machine compliance. Declaration of Conformity. | PDF. Lodged with VS facilities management. | VS facilities, CoffCo compliance | Reissued on any hardware change requiring re-test. |

## 6.2 Engineering Drawings and CAD

- All CoffCo machine CAD models and engineering drawings held in CoffCo PLM (Product Lifecycle Management) system. Access restricted to CoffCo engineering team and approved subcontractors.

- Assembly drawings (2D PDF) provided to regional depot for assembly-swap reference. Drawings version-stamped. Superseded drawings archived but not deleted.

- Schematics (electrical wiring, fluid circuit) held in PLM. PDF versions on diagnostic tablet for field engineer reference during fault diagnosis.

- VS does not receive CAD or detailed drawings — these contain CoffCo IP. VS receives only the User Manual, Housekeeping Guide, FIDA Manual, and certification documents.

## 6.3 Data Management and Configuration Control

| Data Management Element | CoffCo Approach |
| --- | --- |
| Version control (documents) | All technical publications version-controlled in CoffCo CMS. Version number format: V[major].[minor]. Major increment on substantive procedure change; minor increment on editorial correction. Previous versions archived. QR code links always resolve to current version. |
| Version control (firmware) | Firmware version-controlled in CoffCo Git repository. Tag per release. Each machine's firmware version recorded in cloud platform asset register. OTA update includes version verification step. |
| Configuration baseline | Commissioning record establishes baseline for each machine: serial number, firmware version, water hardness calibration, hardware configuration, date of commissioning. Any change to baseline (firmware update, assembly swap) recorded in cloud platform. |
| Accessibility | User-facing manuals publicly accessible via QR code (no login). FIDA Maintenance Manual login-gated (VS maintenance credentials issued at on-site training). Field Service Manual restricted to diagnostic tablet (CoffCo MDM-managed devices only). |
| Compliance standards referenced | IEC 60335-1 (electrical safety); IEC 60529 (IPX2 ingress protection); IEC 61000-4-5 (surge immunity); EN 55014 (EMC Class B); HACCP (food safety); GDPR (data protection); WEEE Directive 2012/19/EU (end-of-life); RoHS Directive 2011/65/EU (hazardous substances). |

# §7. Training

| 7 Training | TNA ¦ Operator/Maintainer training ¦ E-learning ¦ On-the-job ¦ Simulators |
| --- | --- |

## 7.1 Training Needs Analysis

| Stakeholder | Baseline Assumption | Training Gap | Training Output |
| --- | --- | --- | --- |
| Hotel room guest | Familiar with coffee machines generally. | CoffCo-specific operating sequence and consumable format only. | User Manual (web via QR code). No formal training session required (SRS 2.1). |
| VS housekeeping | Trained for basic cleaning as part of existing role. | CoffCo cleaning procedure; 7 status indicator states; consumable restocking; HACCP milk rule; fault escalation. | Housekeeping Guide (web + QR); 45-min on-site training by CoffCo field engineer at commissioning. |
| VS maintenance team | Capable of basic mechanical and electrical maintenance and repairs. | FIDA fault structure; fault codes; safe power isolation; assembly identification; escalation threshold. | FIDA Maintenance Manual (web, login-gated); 60-min on-site training by CoffCo field engineer at commissioning. |
| CoffCo field engineer | Engineering background (mechanical or electrical). | CoffCo-specific: assembly-swap procedures; safety interlock testing; HACCP cleaning; diagnostic tablet; IEC 60335 awareness; FIDA procedures. | 2-day Field Service qualification at CoffCo training centre. Annual refresher (half day). |
| CoffCo cloud ops team | IT or software operations background. | CoffCo cloud platform; telemetry event types and triage; OTA firmware management; fleet analytics. | Cloud ops training programme (1 day, delivered by CoffCo engineering). Platform user guide. |
| VS property managers | Hotel management background. | Cloud dashboard: machine fleet status, alert types, escalation SLA. | 30-min cloud dashboard orientation by CoffCo account manager at go-live. |

## 7.2 Delivery Methods

| Method | Used For | Detail |
| --- | --- | --- |
| E-learning (web-hosted) | User Manual, Housekeeping Guide, FIDA Maintenance Manual | Online manuals with embedded interactive diagrams, step-by-step animations, and video walkthroughs. Multi-language support. Accessible via QR code on machine or Quick-Reference Card. No login required for User Manual and Housekeeping Guide. |
| Classroom (at training centre) | Field engineer 2-day qualification (theory component); cloud ops 1-day training | Theory: IEC 60335 awareness, HACCP principles, FIDA methodology, fault code taxonomy, safety case overview. 12-seat classroom at CoffCo regional training centre. |
| On-the-job / practical (at training centre) | Field engineer 2-day qualification (practical component) | Assembly-swap practical on representative machines. Calibrated instrument use (pressure gauge, thermocouple). Fault injection simulation (WDT, dry-fire, PRV). HACCP cleaning practice. |
| On-site training (at hotel property) | VS housekeeping (45 min); VS maintenance team (60 min) | Delivered by CoffCo field engineer at commissioning of each property. Uses installed, commissioned machine. Attendance recorded. Sign-off by VS property coordinator. |
| Simulator (diagnostic tablet) | VS maintenance team FIDA familiarisation; cloud ops triage training | Pre-loaded simulated fault scenarios on diagnostic tablet. No live machine required. Allows maintenance team to practice FIDA fault navigation before encountering a real fault. |

# §8. Personnel

| 8 Personnel | Workforce planning ¦ Skill requirements ¦ Human factors ¦ Recruitment and retention |
| --- | --- |

## 8.1 Workforce Planning

| Role | Employment Type | Headcount (per region) | Certification / Qualification | Scaling Basis |
| --- | --- | --- | --- | --- |
| CoffCo Field Engineer | CoffCo employed or contracted | 2 minimum at go-live; scale at 1 FE per 50 machines deployed | 2-day CoffCo Field Service qualification. Annual refresher. Basic DBS check. Calibration-checked tooling. | 1 FE per 50 machines is the planning rule (1 annual PM-03 per machine per year; FE can complete approx. 3 per day = 50 machines per year at 2 days/week field time, allowing for corrective callouts). |
| CoffCo Cloud Ops Analyst | CoffCo employed | 2–4 FTE for 24/7 coverage (2 shifts, minimum 2 per shift if alert volume justifies) | CoffCo cloud platform training (1 day). GDPR awareness. Cyber access control briefing. | Scale with fleet size. Below 100 machines: 2 FTE. Above 500 machines: dedicated ops team. |
| CoffCo Regional Depot Technician | CoffCo employed | 1 per regional depot | Assembly refurbishment training (1 day at training centre). HACCP food handler certificate. | 1 depot technician per regional depot. Scales if assembly throughput exceeds 2 swaps/day. |
| CoffCo Account Manager | CoffCo employed | 1 per customer account (Vacation Stables = 1) | Product knowledge qualification. CoffCo NDA. No technical qualification required. | 1 per customer account. |
| VS Housekeeping (trained for CoffCo) | VS employed | Existing headcount — no new hires | CoffCo 45-min onboarding. HACCP awareness briefing. | No additional headcount: onboarding absorbed into existing housekeeping routine. |
| VS Maintenance Team (trained for CoffCo) | VS employed | Existing maintenance headcount | CoffCo 60-min on-site FIDA training. | No additional headcount for CoffCo. Existing hotel maintenance team absorbs first-line fault response. |

## 8.2 Human Factors — Field Engineer and Maintenance Team

| Human Factors Element | CoffCo Approach |
| --- | --- |
| Ergonomics — assembly swap | Thermal Assembly maximum 4 kg one-person lift. All panels accessed from front or top — no under-machine access required. Anti-tamper screws located at accessible heights. No kneeling or sustained awkward postures for routine PM-03. |
| Ergonomics — hotel room working | Machine located on hotel desk at standing/seated working height. Field engineer does not need to move furniture for routine service. Lighting: standard hotel room lighting is adequate; portable lamp carried for poorly lit hotel rooms. |
| Safety — electrical isolation | Mains power isolated at hotel room socket before any chassis access. Fuse panel accessible to field engineer only (SRS 3.11). IEC 60335 awareness briefing in FE qualification. Lock-out procedure: unplug and confirm no power before removing any panel. |
| Safety — thermal hazard | Field engineer does not work on live thermal assembly under operating temperature. Cool-down period: 30 minutes after power-off before Thermal Assembly access. Calibrated thermocouple confirms surface temperature ≤48°C (SRS 3.10) before any surface contact. |
| Safety — HACCP handling | Food-grade disposable gloves for fluid path contact. Sealed HACCP bag for Thermal Assembly transit. Food-grade cleaning agent confirmed non-reactive with skin. COSHH assessment for descale fluid completed by CoffCo and shared with field engineers. |
| Error prevention — FIDA format | FIDA maintenance procedures are structured to prevent action before diagnosis. Field engineer cannot reach Action step without completing Diagnosis. Tier boundary clearly stated in each FIDA entry: 'VS maintenance team — stop here; escalate to CoffCo field engineer'. |
| Error prevention — diagnostic tablet | Diagnostic tablet guides field engineer step-by-step through assembly swap with visual reference. Assembly is keyed (mechanical) to prevent installation in wrong orientation. Firmware re-registration is mandatory step on Control Assembly swap — tablet prompts before closing work order. |

## 8.3 Recruitment and Retention

- Field engineers are the critical resource for CoffCo IPS delivery. Target profile: HNC/HND in electrical or mechanical engineering, or equivalent experience. Electrical bias preferred (IEC 60335 electrical safety work). HACCP food handler certificate desirable.

- Career framework: Junior FE (0–2 years, supervised) → FE (2+ years, independent) → Senior FE (5+ years, Tier 3 escalation, training delivery). Senior FEs can progress to CoffCo engineering team.

- Retention: competitive day rates for contracted FEs; mileage and accommodation covered for travel-heavy regions; diagnostic tablet and tooling provided (not personal equipment). Annual refresher training maintains engagement and skills currency.

- Cloud ops analysts: graduate-level IT/software operations profile. Shift-premium pay for out-of-hours coverage. Progression into CoffCo platform engineering team.

# §9. Supply Support

| 9 Supply Support | Provisioning ¦ Inventory ¦ Rotables ¦ Procurement ¦ Obsolescence |
| --- | --- |

## 9.1 Supported Repair List (SRL)

| Item | Class | Fail Rank | Criticality | Lead Time | Min Stock (per region) | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Thermal Assembly | Rotable | 1 | CRITICAL | 5 days | 2 | Primary failure concentration. Depot holds 2 at all times. Includes PRV and seals. Refurbishment SLA: 10 days after receipt. |
| Control Assembly | Rotable | 4 | HIGH | 5 days | 1 | NTC holder failure risk (Fan et al. manufacturing defect). Firmware re-registration required after swap. |
| PSU + plug modules (all 3 variants) | Rotable | FTA | HIGH | 3 days | 2 | Hold all 3 regional plug variants. Refurbishment SLA: 7 days. At PM-07 (Year 5): mandatory electrical integrity check (insulation resistance test per IEC 60335-1). If COTS PSU supplier states design life of 5 years or less, treat as a hard replacement trigger - obtain supplier MTBF and design life data at procurement stage. |
| FKM seal kit (fluid path) | Consumable | 5 | HIGH | 1 day | 5 kits | Annual replacement at PM-03. FKM/Viton specified for thermal resistance. 24-month shelf life. |
| Pressure relief valve (PRV) | Consumable | 2 | HIGH | 3 days | 3 | Replace at each annual service or SAFE-SHUTDOWN. Do not service in field. |
| NTC holder (temperature sensor mount) | Consumable | 4 | MEDIUM | 3 days | 5 | Fan et al. manufacturing defect risk. Visual inspect at PM-04. Upgrade to PPS (SDD V1.2 action). |
| Descale fluid (food-grade, 1L dose) | Consumable | Scale driver | HIGH | 1 day | 12 doses/property/year | Stock calibrated to property water hardness. 24-month shelf life. |
| Mains fuse (T-type, 5A and 10A) | Consumable | Overcurrent | HIGH | 1 day | 10 per region | Multiple ratings. Field engineer access only. Stock both UK/EU and US ratings per region. |
| Coffee consumables | Operational consumable | N/A | CRITICAL (ops) | Standing order | 20 brews/room at installation | Standing order per property. Demand 2/day/room. FIFO. Demand-driven replenishment. |
| Pump strainer / filter | Consumable | 1 (scale-related) | MEDIUM | 2 days | 3 per region | Replace annually at PM-03. Sized to Thermal Assembly pump specification (SDD V1.2 action). |
| Quick-Reference Card (laminated) | Consumable | N/A | LOW | 1 day | 5/property | Replace if damaged or procedure changes. Held at depot. |

## 9.2 Rotable Cycle Management

Rotable assets (Thermal Assembly, Control Assembly, PSU) cycle between three states. The cloud platform MMS tracks each rotable by serial number through its full cycle.

| State | Description | MMS Action |
| --- | --- | --- |
| Available | Refurbished, tested, and ready at regional depot. Packed in transit case. | MMS: status = AVAILABLE. Depot stock count updated. Below minimum: reorder triggered. |
| Dispatched | Collected by field engineer for assembly swap. In transit or fitted in machine. | MMS: status = DISPATCHED. Linked to work order and machine serial number. |
| Returned — awaiting assessment | Removed from machine. Field engineer has returned to depot in return transit case. | MMS: status = RETURNED. Fault description from field engineer logged. Assessment by depot technician within 2 working days. |
| Under repair | Depot technician refurbishing: disassembly, component replacement, test brew (Thermal), re-test (Control), PAT (PSU), HACCP clean (Thermal). | MMS: status = UNDER REPAIR. Target refurbishment SLA: 10 days (Thermal), 7 days (Control), 7 days (PSU). |
| Rejected — beyond economical repair | Damage beyond refurbishment. Disposed of per WEEE directive. Component parts harvested where possible. | MMS: status = BER. Dispose of per WEEE. Record for fleet failure trend analysis. |

## 9.3 Obsolescence Management

| Obsolescence Risk | Mitigation |
| --- | --- |
| COTS component obsolescence (pump, PRV, NTC, WDT IC) | Multi-source all safety-critical COTS components at design stage (minimum 2 approved suppliers). Maintain Approved Equivalent List (AEL) for each COTS part. AEL reviewed annually. Last-Time-Buy (LTB) option exercised if a primary supplier withdraws — quantity calculated from remaining fleet life × annual replacement rate. |
| Firmware dependency (hardware/software compatibility) | Firmware version matrix maintained: each firmware version tested against all deployed hardware generations. Hardware generation recorded in cloud platform asset register per machine. New firmware release includes backward-compatibility statement. |
| Regulatory standard evolution (IEC 60335, WEEE, RoHS) | CoffCo compliance team monitors regulatory changes in UK, EU, and US markets. 18-month lead time assumed for re-certification if standard changes. Type-test certificate reissued if hardware change required. |
| Diagnostic tablet platform obsolescence | Diagnostic tablet hardware and OS refreshed at minimum 4-year cycle. Field Service Manual and FIDA content maintained as platform-agnostic web content to reduce migration risk. |
| Cloud platform technology evolution | Cloud platform built on standard REST API. Machine telemetry protocol documented and version-controlled. Platform upgrade must not break deployed machine firmware compatibility — breaking changes managed through versioned API. |

# §10. Engineering Support

| 10 Engineering Support | Supportability ¦ Configuration management ¦ Field data feedback ¦ Engineering change |
| --- | --- |

## 10.1 Design for Supportability

- Modular physical architecture (SP-02): assembly-swap model achieves MTTR ≤4 h without requiring component-level repair in the field. Each physical element is independently replaceable.

- Tool-free housekeeping access (SRS 2.5): water reservoir, drip tray, and consumable chamber all accessible without tools. Tier 1 maintenance requires zero specialist equipment.

- Accessibility: all Tier 2 serviceable panels accessed from front or top. No machine movement required for any routine maintenance task.

- Life cycle cost considerations: assembly-swap model has higher upfront spares cost but lower MTTR and lower field engineer skill requirement than component-level repair. LCC modelling confirms swap model is preferred for a hotel room deployment context where minimising downtime is the primary commercial driver.

## 10.2 Configuration Management

| CM Element | CoffCo Approach |
| --- | --- |
| Configuration baseline | Established at commissioning. Commissioning record defines: machine serial number, hardware configuration, firmware version, water hardness calibration, plug module variant, installation date, property/room assignment. Stored in cloud platform asset register. |
| Change recording | Any deviation from baseline is recorded: firmware update (version, date, cloud ops authorisation); assembly swap (assembly serial number in, serial number out, reason, field engineer ID, date); water hardness recalibration. |
| Design reviews | Quarterly design review of fleet telemetry data against FMECA/HAZID predictions. Comparison of observed fault frequencies against Fan et al. failure mode frequencies. Informs SDD and FMECA updates. |
| Field data integration | Fan et al. (2009) external data is the precedent for this approach. Ongoing: cloud ops fleet analytics (PM-06) generates quarterly field data reports. Reports reviewed by CoffCo engineering. Significant deviations from expected failure frequencies trigger a design review. |
| Feedback loop | Field data → FMECA update → IPS Plan update (this document) → SDD update → training update. Cycle time target: 90 days from identification of significant field anomaly to updated IPS Plan issue. |

## 10.3 Engineering Change Process

An Engineering Change Notice (ECN) is raised whenever a design change affects the IPS — for example, when a new COTS component is approved as an equivalent, when a firmware change affects a maintenance procedure, or when a field failure trend requires a design modification.

| Step | Activity | Detail | Owner |
| --- | --- | --- | --- |
| ECN-1 | Identify change | Field data anomaly, FMECA revision, regulatory change, or COTS obsolescence triggers change request. Categorise: Safety (P1 — immediate), Supportability (P2 — next release), Enhancement (P3 — planned). | CoffCo engineering or IPS team |
| ECN-2 | Impact assessment | Assess impact on: SDD, FMECA, IPS Plan, training materials, Field Service Manual, FIDA procedures, spare part specifications, type-test certificates. Safety changes require SDA approval before implementation. | CoffCo engineering + SDA (for safety changes) |
| ECN-3 | Document update | Update affected documents. Issue new version. Update cloud platform asset register if hardware change. Update Technical Publications CMS. | CoffCo engineering + technical publications |
| ECN-4 | Training update | If procedure changes: update FIDA Maintenance Manual and Field Service Manual before deploying change to fleet. Brief field engineers via diagnostic tablet notification. | CoffCo training + cloud ops |
| ECN-5 | Implementation and close | Implement change across fleet. Record completion in cloud platform. Close ECN. Archive previous baseline documents. | CoffCo field engineers + cloud ops |

## 10.4 Open Actions from External Failure Analysis (Fan et al. 2009)

| Ref | Action | Detail | Owner / When |
| --- | --- | --- | --- |
| OA-IPS-01 | NTC holder material upgrade | Upgrade NTC holder specification from PP plastic to PPS material (Fan et al.: PP holder perforates at ~100°C due to poor processing). Add to SDD V1.2. Incoming inspection for NTC holders at manufacture. | CoffCo engineering / SDD V1.2 |
| OA-IPS-02 | Pump strainer specification | Add pump strainer/filter to Thermal Assembly design to prevent debris clogging (ranked #1 failure mode in Fan et al. FMEA). Add to SDD V1.2 and SRL. | CoffCo engineering / SDD V1.2 |
| OA-IPS-03 | Fluid path seal specification | Specify FKM (Viton) elastomers for all fluid path O-rings and gaskets in SDD V1.2. Annual seal replacement confirmed in PM-03 schedule. | CoffCo engineering / SDD V1.2 |
| OA-IPS-04 | Depot bench pump | Procure calibrated bench pump (0–5 bar) for regional depot PRV testing of returned assemblies. Required for rotable refurbishment quality check. | CoffCo depot manager / Before first estate rollout |
| OA-IPS-05 | Fleet analytics baseline | Establish baseline fault frequency expectations from Fan et al. FMEA failure mode frequencies. Use as comparator in PM-06 quarterly fleet review from first quarter of operation. | CoffCo cloud ops + engineering / At go-live |